ECE-161C MODEM DESIGN, SPRING 2021

MEDIUM LENGTH (BUT EASY) FINAL PROJECT

A MATLAB PROGRAMMING PROBLEM AND LEARNING OPPORTUNITY

A. Implement  an  OFDM  Modulator  as  a  128  point  scaled IFFT  with  54  occupied frequency  bins.  In  particular,  bins  –27-to-1  and  +1  to  +27,  (bin  0  empty).  Each bin is 16-QAM.  The IFFT divides the output signal by 128 which is too much attenuation. You will have to scale the IFFT to set the peak amplitude of the time series to be in the amplitude range 0.5 to 1.0.  The  successive OFDM symbol time  series  formed  at  the  output  of  the  transform  is  separated by  a  guard  interval  of  32  zero  valued  samples  located  at  the  beginning  of  each OFDM symbol. In this part, the guard intervals do not contain cyclic prefaces. Form an OFDM packet composed of 100 OFDM symbols. In two out of three subplots plot the real part and imaginary part of the time signal spanning 4-symbol durations. In the third subplot plot the power spectrum of the time series formed by a windowed 2048 point FFT. Comment on the side lobe structure of the spectrum. Use a Kaiser window with beta equal to 8.3, a value chosen to obtain -60 dB stopband side lobes.

B. Demodulate  the  OFDM  packet  and  in one subplot display  the  (time)  overlaid  real  part  of  the successive transforms in the (frequency) constellation diagram when there is no channel distortion. The gain we used to scale the IFFT in the modulator must be removed as an attenuation in the FFT demodulator to keep the constellation points in the unit square. In the second subplot plot the constellation points from all frequency bins in the OFDM symbols.

C. Pass the 100 OFDM symbols in our packet through a channel with impulse response 
[1   0   0.2   0   0   0   0   0   0   j*0.1   ]

Now repeat part B and demodulate the channel distorted received OFDM packet and in one subplot display  the  (time)  overlaid  real  part  of  the successive transforms in the (frequency) constellation diagram when there is channel distortion. The gain we used to scale the IFFT in the modulator must be removed as an attenuation in the FFT demodulator to keep the constellation points in the unit square. In the second subplot plot the constellation points from all frequency bins in the OFDM symbols.

D. Replace  the  32-sample  guard  interval  with  a  32  sample  cyclic  prefix  taken  from the  last  32  samples  of  each  OFDM  symbol.  Pass this new packet through the  same channel. Now repeat part B and demodulate the channel distorted received OFDM packet and in one subplot display the (time) overlaid real  part  of  the successive transforms in the (frequency) constellation diagram when there is channel distortion. The gain we used to scale the IFFT in the modulator must be removed as an attenuation in the FFT demodulator to keep the constellation points in the unit square. In the second subplot plot the constellation points from all frequency bins in the OFDM symbols.

E. Insert two long preambles to the beginning of the 100 symbol modulated packet formed with the 32 sample cyclic prefix. The long preamble is formed by random QPSK constellation points in all 52 occupied OFDM frequency bins. Perform the 128 point IFFT and append two copies of the 128 point IFFT. Then take the last 64 samples of the second copy and move it to be the double length cyclic prefix of the long preamble. This trick avoids any discontinuities in the double length preamble. Pass the long preamble through the channel. Demodulate the two 128 point sequences. Each bin of the 52 bins had a known input amplitude and phase when it left the modulator. We have measured the gain and phase of each bit at the output of the channel. The  relationship  between  channel  and  received  signal  at  each frequency  is  of  the  form  Frcvd(K)=Fxmtd(k)*  Fchannel(k).  From  this  relationship  we  can estimate  the  channel  and  remove  it from the  received  signal.

Form 2 subplots. In the first show the 128 point FFT magnitude spectrum of the channel, the magnitude spectrum of the long preamble, and the magnitude spectrum of the received symbol from the channel. In the second show the 128 point FFT magnitude spectrum of the channel and the channel estimate obtained from the long preamble.

F. From here on, each successive symbol is received through the channel. We multiply each received constellation point by the inverse of the channel response at each frequency to obtain the equalized received constellation points. Apply the equalizer inverse channel gain to each received symbol and plot on two subplots plot the equalized time overlaid  real  part  of  the successive transforms in the (frequency) constellation diagram. The gain we used to scale the IFFT in the modulator must still be removed as an attenuation in the FFT demodulator to keep the constellation points in the unit square. In the second subplot plot the equalized constellation points from all frequency bins in the OFDM symbols.

G. Insert  50  samples  of  delay  and  a  single  OFDM  symbol  length  preamble  at  the beginning of the OFDM packet formed by zero packing the spectrum 1-to-4. Only insert random QPSK data  in  bins 4,   8, 12,...., 24.  This symbol also has an appended cyclic prefix. Add white noise of standard deviation 0.01 to the delay and preamble. Spin the  data  being  delivered  through  the  channel  at  3  degrees  per  sample.  Use  a  32  sample delay  line  and  form  the  cross  correlation  between  input  and  output  and  form  the autocorrelation  at  the  output  of  the  delay  line.  Take the ratio of the auto and cross correlation sequences.  Verify  that  the  sample  by  sample  ratio  of  the  cross  to  the  auto correlation  identifies  the  start  of  the  OFDM  signal.  Present a figure showing magnitude of cross, auto, and ratio.  Also  show  the  angle  obtained  at  the  output  of  the  32  sample averager processing the cross correlation. Verify this is a reasonable estimate of the input rotation rate. Present a figure showing output of 32-point averager and the estimated rate of rotation.  This estimated rotation rate would normally be used to de-spin subsequent signal samples following this short preamble. Since we did not include pilots in this project, we will not actually use the estimate to de-spin the input signal

A bit of assistance follows

%40 MHz sample rate, 1/40 usec/sample, 160 samples/symbol, 4 usec duration

% 1/40 usec/sample, 128 samples/symbol, 3.2 usec transform segment

% spectral channel spacing 1/3.2 usec = 312.5 kHz

x0=[]; % empty array for OFDM with Guard interval

y0=[]; % empty array for OFDM with cyclic prefices

for k=1:100

fxx=zeros(1,128);

dat=(floor(4*rand(1,54))-1.5)/1.5+j*(floor(4*rand(1,54))-1.5)/1.5;

fxx(65+(-27:27))=[dat(1:27) 0 dat(28:54)];

xx=8*ifft(fftshift(fxx));

x0=[x0 zeros(1,32) xx];

y0=[y0 xx(97:128) xx];

end

figure(1)

subplot(2,1,1)

plot(0:640, real(x0(1:641)),'b','linewidth',2)

hold on

for k=1:160:640

plot(k:k+31,real(x0(k:k+31)),'r','linewidth',2)

end

hold off

grid on

set(gca,'XTick',[0:32:640])

axis([0 640 -1.2 1.2])

title('Real Part 128 Point OFDM Signal, with 32 Sample Guard Intervals')

xlabel('Time Index')

ylabel('Amplitude')

subplot(2,1,2)

plot(0:640, real(y0(1:641)),'b','linewidth',2)

hold on

for k=1:160:640

plot(k:k+31,real(y0(k:k+31)),'r','linewidth',2)

end

hold off

grid on

set(gca,'XTick',[0:32:640])

axis([0 640 -1.2 1.2])

title('Real Part 128 Point OFDM Signal, with 32 Sample Cyclic Prefices')

xlabel('Time Index')

ylabel('Amplitude')

figure(2)

subplot(2,1,1)

plot(0,0)

hold on

for n=1:160:16000-160

x1=x0(n:n+159);

fx1=fftshift(fft(x1(33:160)))/8;

plot(-64:63,real(fx1),'ro','linewidth',2,'markersize',6)

end

hold off

grid on

axis([-32 32 -1.5 1.5])

set(gca,'XTick',[-64:8:64])

title('Real Part, FFT Transform Segments of OFDM Symbols')

subplot(2,1,2)

plot(0,0)

hold on

for n=1:160:16000-160

x1=x0(n:n+159);

fx1=fftshift(fft(x1(33:160)))/8;

plot(fx1,'ro','linewidth',2,'markersize',6)

end

hold off

grid on

axis('equal')

axis([-1.5 1.5 -1.5 1.5])